TASK-4  SIMPLE IOT : ESP32 WEB DASHBOARD

ESP32 SKETCH :

#include <WiFi.h>

#include <WebServer.h>

#include <DHT.h>

#define DHTPIN 4

#define DHTTYPE DHT11   // Change to DHT22 if needed

#define LED_PIN 2

DHT dht(DHTPIN, DHTTYPE);

WebServer server(80);

// ESP32 Access Point credentials

const char* ssid = "ESP32_Dashboard";

const char* password = "12345678";

bool ledState = false;

// HTML Dashboard

String webpage = R"rawliteral(

<!DOCTYPE html>

<html>

<head>

<title>ESP32 IoT Dashboard</title>

<meta name="viewport" content="width=device-width, initial-scale=1">

<style>

body {

margin: 0;

font-family: 'Segoe UI', Tahoma, Geneva, Verdana, sans-serif;

background: linear-gradient(135deg, #667eea, #764ba2);

color: #333;

}

header {

background: rgba(255,255,255,0.15);

padding: 20px;

text-align: center;

color: white;

font-size: 24px;

font-weight: 600;

}

.container {

display: grid;

grid-template-columns: repeat(auto-fit, minmax(250px, 1fr));

gap: 20px;

padding: 25px;

}

.card {

background: white;

border-radius: 15px;

padding: 25px;

box-shadow: 0 10px 25px rgba(0,0,0,0.15);

text-align: center;

}

.label {

font-size: 18px;

color: #666;

}

.value {

font-size: 40px;

font-weight: bold;

margin-top: 10px;

}

.temp {

color: #e74c3c;

}

.hum {

color: #3498db;

}

.btn {

padding: 15px;

width: 100%;

font-size: 18px;

border: none;

border-radius: 10px;

cursor: pointer;

transition: 0.3s;

}

.btn-off {

background: #2ecc71;

color: white;

}

.btn-on {

background: #e74c3c;

color: white;

}

footer {

text-align: center;

padding: 15px;

font-size: 14px;

color: white;

opacity: 0.8;

}

</style>

</head>

<body>

<header>ESP32 IoT Web Dashboard</header>

<div class="container">

<div class="card">

<div class="label">Temperature</div>

<div id="temp" class="value temp">-- °C</div>

</div>

<div class="card">

<div class="label">Humidity</div>

<div id="hum" class="value hum">-- %</div>

</div>

<div class="card">

<div class="label">Device Control</div>

<button id="ledBtn" class="btn btn-off" onclick="toggleLED()">LED OFF</button>

</div>

</div>

<footer>

Powered by ESP32 • Real-Time IoT Monitoring

</footer>

<script>

let ledState = false;

setInterval(() => {

fetch("/sensor")

.then(response => response.json())

.then(data => {

document.getElementById("temp").innerHTML = data.temp + " °C";

document.getElementById("hum").innerHTML = data.hum + " %";

});

}, 2000);

function toggleLED() {

fetch("/led");

ledState = !ledState;

let btn = document.getElementById("ledBtn");

if (ledState) {

btn.innerHTML = "LED ON";

btn.className = "btn btn-on";

} else {

btn.innerHTML = "LED OFF";

btn.className = "btn btn-off";

}

}

</script>

</body>

</html>

)rawliteral";

void handleRoot() {

server.send(200, "text/html", webpage);

}

void handleSensor() {

float temp = dht.readTemperature();

float hum = dht.readHumidity();

String json = "{";

json += "\"temp\":" + String(temp) + ",";

json += "\"hum\":" + String(hum);

json += "}";

server.send(200, "application/json", json);

}

void handleLED() {

ledState = !ledState;

digitalWrite(LED_PIN, ledState);

server.send(200, "text/plain", "OK");

}

void setup() {

Serial.begin(115200);

pinMode(LED_PIN, OUTPUT);

dht.begin();

// Start ESP32 Access Point

WiFi.softAP(ssid, password);

Serial.println("ESP32 AP Started");

Serial.println(WiFi.softAPIP());

server.on("/", handleRoot);

server.on("/sensor", handleSensor);

server.on("/led", handleLED);

server.begin();

}

void loop() {

server.handleClient();

}

SHORT DEMO NOTES :

The ESP32 hosts a local web server that displays live sensor readings on a web dashboard.
The user can remotely toggle an LED/relay using the dashboard buttons.
All communication happens over Wi-Fi using HTTP requests, without any external server.

The ESP32 acts as both a data acquisition unit and a web server.
Sensor values are periodically read and sent to the web dashboard for real-time display.
User actions on the dashboard (button clicks) are received by the ESP32 as HTTP requests and mapped to GPIO control.
This demonstrates a complete end-to-end embedded IoT system, combining sensing, processing, communication, and control.

DASBOARD SNAPSHOT :